Heaven’s light is our guide

RAJSHAHI UNIVERSITY OF ENGINEERING & TECHNOLOGY

Department of Mechatronics Engineering

Lab Report

Course No	: 	MTE 1102

Course Title	: 	Mechatronic System Sessional

Experiment No	: 	01

Experiment Title	: 	Study of the basic equipment of Mechatronics System.

Submitted To:

Prangon Das

Lecturer

Department of Mechatronics Engineering

Rajshahi University of Engineering & Technology, Rajshahi.

Subrata Kumar Sarker

Lecturer

Department of Mechatronics Engineering

Rajshahi University of Engineering & Technology, Rajshahi.

Submitted By:

Group-5

Mamun Or Rashid 	2008025

Nazib Abrar 	2008026

- M. Al Meraz 	2008027

- Md. Akkhor Hasan 	2008028

- Md. Emdadul Haque Emon 	2008029

- Md. Maherul Islam Maruf 	2008030

Date Of Experiment	: 16 March, 2022

Date Of Submission	: 28 March, 2022

Experiment No.:  01

Name of the Experiment: Study of basic equipment of Mechatronics.

Introduction:

The term mechatronics system encompasses a myriad of devices and systems. Physically, a mechatronic system is composed of four prime components. They are sensors, actuators, controllers, and mechanical components. Input devices such as gas sensor, ultrasonic sensor, temperature sensor, keypad, etc. and  output devices such as seven segment display, LCD display, pneumatic actuator, servo motor, stepper motor, DC motor etc. and some controllers such as Arduino, PLC, etc. are the basic components of Mechatronic System.

The figure shows a schematic diagram of a mechatronic system integrated with all the above components.

MTE 1102 course offers hands-on learning with advanced modern types of equipment. Artificial Intelligence and Control System Lab, Robotics and Automation Lab, Computer & Simulation Lab are the three modern and sophisticated laboratories equipped with  different instruments and machines to enhance theoretical knowledge with practical experience.

Objectives:

- To learn about the basic mechatronics systems.

- To identify the input devices, output devices, and control systems.

- To visualize the practical working areas and the applications of these basic mechatronics equipment.

- To know the various pneumatic components used in a pneumatic system and to understand their workings.

Input Devices:

- Gas Sensor:

Figure 1: MQ-7 Gas Sensor and MQ-135 Gas Sensor.

Specifications:

| Parameter name | Technical Condition (MQ-7) | Technical Condition (MQ-135) |
| --- | --- | --- |
| Circuit voltage | 5V±0.1 | 5V±0.1 |
| Heating voltage (high) | 5V±0.1 | 5V±0.1 |
| Load resistance | Can adjust | Can adjust |
| Heating resistance | 33Ω±5% | 33Ω±5% |
| Heating consumption | About 350mW | less than 800mw |

Table 1: Specifications of MQ-7 and MQ-135 Gas Sensor

Applications:

- Gas sensors are used in protection of individuals working in hazardous environments where the presence of dangerous gases can be found.

- Gas sensors are used in monitoring indoor air quality.

- Aerospace environments utilize gas sensors specifically to monitor for in-flight conditions and air quality maintenance to ensure crew productivity, as well as overall passenger comfort.

- Gas sensors are also used in agricultural sector to ensure suitable environment in greenhouses for the growth of crops.

- Ultrasonic Sensor:

Figure 2: HC-SR04 Ultrasonic Distance Sensor.

Specifications:

| Parameter name | Technical condition |
| --- | --- |
| Working Voltage | DC 5 V |
| Working Current | 15mA |
| Working Frequency | 40Hz |
| Max Range | 4m |
| Min Range | 2cm |
| Measuring Angle | 15 degree |
| Trigger Input Signal | 10uS TTL pulse |
| Echo Output Signal | Input TTL lever signal and the range in proportion |
| Dimension | 45*20*15mm |

Table 2: Specifications of HC-SR04 Ultrasonic Sensor

Applications:

- Ultrasonic sensors are used for measuring the liquid level in various industries for automated control.

- Modern vehicles use ultrasonic sensors to measure the distance between the car and the surrounding elements of the road for understanding the environment and drive accordingly.

- Ultrasonic sensors can be applied to the manufacturing process for automated process control on the factory floor while also being an indispensable tool for companies to maximize efficiency through precise measurement and control.

- Keypad:

Figure 3: COM-16038 4x4 Matrix Membrane Keypad

Specifications:

| Parameter Name | Technical Condition |
| --- | --- |
| Maximum Rating | 24 VDC, 30 mA |
| Interface | 8-pin access to 4x4 matrix |
| Operating temperature | 32 to 122° F |
| Dimensions | Keypad: 2.7 x 3.0 in Cable: .78 x 3.5 in |

Table 3: Specifications of COM-16038 4x4 Matrix Membrane Keypad

Applications:

- These type of keyboards are used as input in control systems where human interaction is needed.

- To implement password protection in any door lock system, this keypad is needed.

- This keypad could be used in electronic voting machines.

- Temperature Sensor:

Figure 4: DFR-0198 Digital Temperature Sensor

Specifications:

| Parameter Name | Technical Condition |
| --- | --- |
| Voltage Range | 3.0V to 5.5V |
| Accuracy | ±0.5°C from -10° to +85°C |
| Usable Temperature Range | -55°C to 125°C |
| Resolution | 9 to 12 bit selectable |

Table 4: DFR-0198 Digital Temperature Sensor

Applications:

- Temperature sensors are mostly used for detection of industrial component’s heat measurement for detecting overheating issues.

- Inside computers, temperature sensors are used to detect overheating of the electronic components and regulate the cooler fan speed accordingly.

- In 3D printers, temperature sensors are used for measuring the temperature of the melted  material for ensuring proper performance and durability of the printed product.

- DC Power Supply:

Figure 5: Protek PL-3003S power supply

Specifications:

| Parameter Name | Technical Condition |
| --- | --- |
| Output Power | 90W |
| Output Voltage | 0-30V |
| Output Current | 0-3A |
| Output Port Type | Single Type |
| Display Type | Back Light 3 Digit LCD |

Table 5: Specifications of Protek PL-3003S power supply

Applications:

- Power supplies are used as precise voltage and current source for a system.

- For testing and fixing broken systems, where variable power source is needed, these type of power sources are used.

Output Devices:

- Servo Motor:

Figure 6: SM-S4303 Continuous Rotation Servo Motor

Specifications:

| Parameter Name | Technical Condition |
| --- | --- |
| Speed | 54 rpm at 6V 43 rpm at 4.8V |
| Stall Torque | 71 oz |
| Lead length | 11 inch |
| Weight | 41 g |

Table 6: Specifications of SM-S4303 Continuous Rotation Servo Motor

Applications:

- Servo motors are used in industrial production robots to work accurately in the manufacture process.

- In digital cameras servo motors are used for auto focusing and self balancing.

- In elevator technology, servo motors are used for proper accurate height and speed adjustments.

- In robotics, servos are used for precise actuation.

- DC Motor:

Figure 7: EG-530AD-2B DC motor

Specifications:

| Parameter Name | Technical Condition |
| --- | --- |
| Speed | 2400 rpm (max) |
| Current | 73mA - 130 mA |
| Starting Torque | 37.2/4.2V |
| Voltage | 6V-12V |

Table 7: Specifications of EG-530AD-2B DC motor

Applications:

- DC motors are used for actuation of robots.

- In automated vehicles, DC motors are used for various safety features.

- In industries, DC motors are used as electronic locomotives.

- In cranes, DC motors are used for hoisting heavy things as these motors have high staring torque capability.

- 16x2 LCD display:

Figure 8: 16x2 LCD display

Specifications:

| Parameter Name | Technical Condition |
| --- | --- |
| Input Voltage | 3.5V |
| Supply Current | 2.5mA |
| Enable Pulse width | 300ns |
| Response time | 250ms |

Table 8: Specifications of a 16x2 LCD display

Applications:

- This type of LCD display is used for displaying output of different sensors.

- These are used for creating user interface of a system.

- These are used extensively in mobile phones, calculators, computers and clocks.

- Dot LED matrix:

Figure 9:  Dot LED Matrix display

Specifications:

| Parameter Name | Technical Condition |
| --- | --- |
| Input Voltage | 5V |
| Supply Current | 320mA |
| LED colour | Red |

Table 9: Dot  LED Matrix display

- Applications:

- Dot LED matrix is used in signboards as a cost-efficient way of advertisement.

- These displays are used in clocks for displaying time.

- These are used in bus-stops and railway stations as departure indicators.

- 7-Segment Display:

Figure 10:  Dot LED Matrix display

Specifications:

| Parameter Name | Technical Condition |
| --- | --- |
| Input Voltage | 3.3V |
| Pins | 10 |
| Average Power Consumption | 86mW |

Table 10: 7-Segment display

- Applications:

- These type of displays are used in showing numerical data output from any device or sensor.

- Calculators, digital clocks, radios, electronics meters use them extensively.

- Oscilloscope:

Figure 11:  Protek-5100 Digital Oscilloscope.

Specifications:

| Parameter Name | Technical Condition |
| --- | --- |
| Bandwidth | 100MHz |
| Channels | 2 channels + 1 Extension |
| Range | 2.5ns/Div-50s/Div |
| Step | 1-2.5-5 order |

Table 11: Protek-5100 Digital Oscilloscope.

- Applications:

- Oscilloscopes are used to show electrical signals and how those signals change over time.

- Primarily these are used for measuring voltage waves..

- This device is used for testing and debugging electric circuits.

Controllers:

- Arduino:

Figure 13: Arduino Uno R3

Specifications:

| Parameter name | Technical Condition |
| --- | --- |
| Micro-controller | ATmega328P |
| Operating Voltage | 5V |
| Input Voltage (recommended) | 7-12V |
| Digital I/O Pins | 14 (of which 6 provide PWM output) |
| PWM Digital I/O Pins | 6 |
| Analog Input Pins | 6 |
| Flash Memory | 32 KB (ATmega328P) of which 0.5 KB used by bootloader |
| SRAM | 2 KB (ATmega328P) |
| EEPROM | 1 KB (ATmega328P) |
| Clock Speed | 16 MHz |

- Applications:

The applications of Arduino Uno mainly involves in Arduino Uno based projects which include the following

- Visitor Alarm in Office using Arduino Uno

- Arduino Uno based Soccer Robot

- Arduino Uno based Automatic Medication Reminder

- Motion Detecting with Static Electricity

- Arduino Uno based Taxi with Digital Fare Meter

- Arduino Uno based Smart Stick

- Robot Car Controlled by Smartphone and Arduino

2. PLC:

Figure 14: SIMATIC S7-1200

Specifications:

| Parameter Name | Technical Condition |
| --- | --- |
| Power Dissipation | 14W |
| Digital Input Current | 4mA input used |
| Bit Memory | 8192 bytes |
| Pulse Catch Input | 14 |
| Onboard Digital I/O | 14 inputs / 10 outputs |
| Onboard Analog I/O | 2 Inputs / 2 Outputs |

Applications:

- Industrial Applications:

- Time and Count-based Control System for an Industrial Machine.

- Temperature Controller or Humidity by using the Sensors Input to the PLC system.

- Fault Detection and Protection of Industrial Machines like an Induction Motor.

- Power Station Applications:

- PLC uses for the Smart Grid System to Monitor and Detect fault conditions.

- It is used in the Power Generation, Transmission, and PLC Distribution System.

- Domestic Applications:

- Water Tank Level Control System.

- Car Washing and Parking System.

- Flashing Light Controlling System.

- Automatic Door Opening/Closing System.

- 3. Sweep Function Generator:

Figure 14: Protek 9205C 0.02 Hz ~ 2 MHz Sweep Function Generator

| Description |  | Specification |
| --- | --- | --- |
| Output | Frequency Range | 0.02Hz to 2MHz (7Ranges) |
|  | Accuracy | Over 1Hz ± Count ± Time Base Error |
|  | Amplitude | 1Vp to 20Vp (Open Circuit), 0.5Vp to 5Vp(50 Ω) |
|  | Attenuation | Fixed 20dB ±1dB, 20dB Continuity Variable |
|  | Impedance | 50Ω ±5% |
| Sine wave | Flatness | Better Than ±3dB ~ 2MHz at Max. Output |
|  | Distortion | 2% (10Hz ~100kH2) |
| Square Wave | Rise / Fall Time | 100ns or Less |
| Triangle Wave | Nonlinearity | 1% to 100kHz |
|  | Symmety Variation | 1:1 ~ 4:1 |
|  | VCF Input | OV ~ 10V DC |
| Sweep function | Mode | Linear |
|  | Width | 10:1 ~ 100:1 |
|  | Time | 0.5Hz(2sec) to 50Hz (20msec) |
| Frequency counter | Range | 10Hz ~ 2MHz |
|  | Accuracy | 0.01% ±1Count |
|  | Max. Input Voltage | 70Vp |
|  | Input Impedanoe | 1MΩ |

Application:

- Used for testing DC power supply

- For testing the delay margin

- Analyze the audio DAC

- To test clock frequency functional range of digital circuits

- Employed for testing and optimization of engine controlling units

- Analyze switching signals of IGBT circuitry

Applications:

- Industrial Robotic Arm:

Figure : FANUC LR-Mate 200ID

Specifications:

| Parameter Name | Technical Condition |
| --- | --- |
| Controlled Axes | 6 |
| Max Payload | 7kg |
| Repeatability | 0.01mm |
| Mounting Method | Floor/Upside Down/Angle |
| Reach | 717 |

Table 6:Specifications of FANUC LR-Mate 200ID

Applications:

- FANUC robots are used in factories around the globe to help manufacturers improve quality, increase capacity and overcome skills shortages.

- This type of robots build products too intricate for human hands to assemble, prepare pharmaceuticals in aseptic conditions and protect workers from dangerous or harmful environments.

- FANUC Robotics produces industrial robots that can be used in wielding automation and robotic assembly applications.

Precautions:

- When observing the lab equipment, careful measures were taken.

- All high voltage parts and connections were kept out of the way from accidental touching.

- Whether the connections with the load were causing sparks was carefully checked.

- Rubber floor mates were used to insulate ourselves from the ground when working in the lab.

- After finishing the observation, all connections to the power supply was removed.

Discussion & Conclusion:

We are able to successfully obtain our objectives of the observation on the basic mechatronics equipment. Like, we have come to learn about some input devices such as gas sensor,ultrasonic sensor,temperature sensor,keypad etc. & output devices such as seven segment display, LCD display, pneumatic actuator, servo motor, Stepper motor, DC motor etc. and some controllers such as arduino, PLC etc.We study on internet and books to find out the specification,application of these device and collect the image of these equipment.So,we also manage to learn about the applications of these basic components in various mechatronics devices and systems.In some case we are able to know how can we drive the device and how the devices run.

However, as a fresher on this site,we have faced so many difficulties.Like we are confused about some devices, whether they are input device or output device or just a controller.We have our hand short of information to make the datasheets of some devices.Despite having all of these problems,we have successfully managed to complete our objectives.

Gaining Knowledge about the basic mechatronics systems and their applications will help us in the future to do bigger and more complicated experiences confidently and correctly.

As a new comer we must know about the basic mechatronic equipments first.And in this experiment we all are able to know the specification,application of some input device,output device,some actuators,controllers and some sensors.